# Dataset Schema: data.csv

AI4I 2020 Predictive Maintenance Dataset

File: data_v0/data.csv
Rows: 10,000 (plus header)
Columns: 14

## Column Definitions

| Column | Type | Description |
| --- | --- | --- |
| UDI | Integer | Unique row/sample identifier (1-10000). |
| Product ID | String | Product serial number, e.g. M14860. Letter prefix encodes the quality variant plus a numeric ID. |
| Type | Categorical (L / M / H) | Product quality variant: Low, Medium, or High. |
| Air temperature [K] | Float | Ambient air temperature, in Kelvin. |
| Process temperature [K] | Float | Process/machine temperature, in Kelvin. |
| Rotational speed [rpm] | Integer | Tool rotational speed, in revolutions per minute. |
| Torque [Nm] | Float | Torque applied, in Newton-meters. |
| Tool wear [min] | Integer | Cumulative tool wear time, in minutes. |
| Machine failure | Binary (0/1) | Overall failure label - 1 if any of the five failure modes below fired. |
| TWF | Binary (0/1) | Tool Wear Failure indicator. |
| HDF | Binary (0/1) | Heat Dissipation Failure indicator. |
| PWF | Binary (0/1) | Power Failure indicator. |
| OSF | Binary (0/1) | Overstrain Failure indicator. |
| RNF | Binary (0/1) | Random Failure indicator. |

## Notes

Machine failure = 1 if and only if at least one of TWF, HDF, PWF, OSF, or RNF equals 1.

Class imbalance is expected: failures are rare relative to normal operation.

Product ID's letter prefix duplicates the Type column, so it is likely redundant as a model feature.

UDI is a row index and carries no predictive signal.